# Preflight Check List

Inside

PLACE KEYS ON GLARE SHEILD

RELEASE control wheel belts

Required papers – on board

Record – Hobbs, tach, check squawks

Avionics Master Off

Check Circuit Breakers

Electrical Switches Off

Master switch ON

CHECK Fuel quantity gauges

CHECK Rotating beacon on tail is operational

CHECK navigational lights are operational (Night Flight)

Check Pitot (In possible Icing)

Master switch OFF

Fuel Selector to right tank

Extend Flaps

Outside/Right Side

Control surfaces – check for interference, free of ice, snow, frost

Hinges – check for interference

Navigation lights – CHECK

Wings – free of ice, snow, frost, leading edge in good condition

Tie Down – remove

Fuel Tank – visually verify fuel level and SECURE CAP

Fuel tank sump – drain and check for water, sediment and proper fuel

Fuel vents – OPEN

Main gear strut – proper inflation of 4.5 in

Tire – check for dry rot, balding, proper inflation 24 psi.

Brake blocks – CHECK

Remove Chocks

Outside/Front

Oil – CHECK LEVEL – min safe quantity 2 quarts – max is 8 quarts (Aeroshell 15W–50)

Dipstick – properly seated (not too tight, but not too loose)

Fuel and Oil – CHECK for leaks

Windshield – Check that it is clean

Cowling – Secure

Inspection covers – Secure

Nose wheel tire – check for dry rot, bald spots and proper inflation 18 psi

Nose gear strut – Proper inflation of 3.25 in.

Remove Chocks

Air inlets – clear of debris

Alternator belt – check tension

Fuel tank sump – drain and check for water, sediment and proper fuel

Outside/Left Side

Brake blocks – CHECK

Tire – check for dry rot, balding, proper inflation 24 psi.

Main gear strut – proper inflation of 4.5 in

Remove Chocks

Fuel vents – OPEN

Fuel Tank – visually verify fuel level and SECURE CAP

Fuel tank sump – drain and check for water, sediment and proper fuel

Pitot head – remove cover – holes clear

Tie Down – remove

Navigation lights – CHECK

Wings – free of ice, snow, frost, leading edge in good condition

Control surfaces – check for interference, free of ice, snow, frost

Hinges – check for interference

Back

Stabilator and Tail  – free of ice, snow, frost

Hinges – check for interference

Tie Down – remove

Tow bar and control locks – stowed

Baggage – stowed properly

Baggage door – close and secure

Flaps up

Fuel Selector to left tank

Nose drain sump – check.

Fuel strainer – emptied and stowed

Seat belts and harness – fasten/adjust and check inertia reel

ARCHER II (PA-28 181) -- NORMAL OPERATING PROCEDURES	N4052F

BEFORE STARTING ENGINE

- Preflight Inspection – COMPLETE

Surrounding Area – CHECK

Seats and Belts – ADJUST / LOCK

Passengers – BRIEFED

Fuel Selector– DESIRED TANK

Brakes – TEST and SET

Flashing Beacon Switch - ON

STARTING ENGINE COLD

- Brakes – SET

- Fuel selector – DESIRED TANK

- Prime -  as required / LOCKED

- Throttle –  OPEN ¼ INCH

- Mixture – FULL RICH

- Carburetor heat – FULL COLD

- Master Switch – ON

- Electric fuel pump – ON

- Propeller Area – CLEAR

- Starter - ENGAGE

- Throttle: 800 – 1200 RPM

- Oil pressure – CHECK

- Avionics Master Switch – ON
Radios – SET & CHECK
Transponder – SQUAWK STBY

- Electrical Equipment – ON

- Fuel Pump OFF

TAXI CHECK

- Taxi area – CLEAR

- Radio – TAXI CLEARANCE or announce intentions

- Brakes - CHECK

- Steering – CHECK

- Ailerons – POSITION for x-wind

- CHK: Mag Compass, DG, Turn Coord.

Ground/Run-up Check

- Brakes Engaged

- Belts and Harnesses - Check

- Empty Seats – belts snugly fastened

- Seat Backs – Erect

- Fight Instruments – SET (attitude Ind, DG, Altimeter)

- Fuel Quantity – CHECK

- Primer – IN and LOCKED

- Elevator/Rudder Trim – AS REQD

- Controls Free / Operational

- Throttle – 2000 RPM

- Magnetos – max drop 175 RPM and max difference 50 RPM

- Vacuum – 5.0” HG +- 0.1

- Oil Temp – Check

- Oil Pressure – Check

- Check fuel pressure

- Annunciator panel – press to test

- Carb. heat ON: Check RPM Drop

- Throttle –IDLE; Check

- Carburetor heat – OFF RPM rise

- Throttle – 800-1200 RPM

- Electric Fuel Pump – ON

- Engine Gauges – Check

- Flaps – AS REQUIRED

- Strobe Lights – ON

- Nav. Lights – ON (Night)

- Doors Latched

- Open Flight Plan & Call for (announce) Takeoff Clearance

- SET CODE & SQUAWK ALTITUDE

- Record Time Off

- Engine is warm for takeoff when throttle can be opened without engine faltering

Normal Takeoff

- Wing flaps – SET 0-10 degrees

- Trim Tab - SET

- Carburetor Heat – COLD

- Throttle – FULL OPEN

- AT 52-65 KIAS - BACK PRESSURE ROTATE TO CLIMB ATTITUDE

CRUISE

- Electric Fuel Pump – OFF once at cruise altitude

- Normal Max Power – 75%

- Power – SET PER POWER TABLE

- Elevator and rudder trim – ADJUST

- Mixture – LEAN ABOVE 5000 FEET

- DG – CHECK / SET

DESCENT

- Throttle – 2500 RPM

- Airspeed – 126 KIAS

- Mixture – RICH

- Carburetor Heat – ON IF REQD

APPROACH AND NORMAL LANDING

- Fuel Selector – FULLEST TANK

- Seat Backs / Belts / Harness

- Electric Fuel Pump – ON

- Mixture –  FULL RICH

- Carburetor Heat – ONLY IN SUSPECTED ICING CONDITIONS

- Flaps – SET – (VFE= 102 KIAS)

- Trim TO 75 KIAS

- Final approach: flaps 40°@66 KIAS

- Touchdown: NOSE UP, FLAPS UP

- Nose down:  GENTLY APPLY BRAKES

AFTER LANDING

- Flaps – UP

- Electric Fuel Pump – OFF

- Carburetor Heat – COLD

- Transponder – STBY

- Strobe Lights – OFF

- Landing / Taxi Lights – OFF 
(as required at night)

- Radio Call

- CLOSE FLIGHT PLAN

STOPPING ENGINE

- Parking break – SET

- Flight Plan – CLOSED

- Avionics Master – OFF

- Electrical Equipment – OFF

- Throttle – CLOSED (IDLE)

- Mixture – IDLE CUTOFF

- Ignition Switch – OFF – Remove keys and place on glare shield

- Master Switch – OFF

- Control Wheel – TIE OFF WITH SEATBELT

- Record – Hobbs, Tach, Squawks

- Clean Cabin

- TIE DOWN AIRPLANE

- PITOT TUBE COVER

- COVER AIRPLANE

| Airspeed IND | V (KIAS) |  |
| --- | --- | --- |
| Stall (Landing) | VSO | 49 |
| Stall (Clean) | VS | 55 |
| Best Angle | VX | 64 |
| Best Rate | VY | 76 |
| Flaps | VFE | 102 |
| Maneuver | VA | 113 |
| Normal | VNO | 125 |
| Never Exceed | VNE | 154 |
| Best Glide |  | 76 |
| Initial Approach Speed |  | 76 |
| Final Approach Speed |  | 66 |

ARCHER II (PA-28 181) – UNUSUAL OPERATING PROCEDURES	N4052F

STARTING ENGINE HOT

- Brakes – SET

- Fuel selector – DESIRED TANK

- Prime -  as required / LOCKED

- Throttle –  OPEN ½  INCH

- Mixture – FULL RICH

- Carburetor heat – FULL COLD

- Master Switch – ON

- Electric fuel pump – ON

- Propeller Area – CLEAR

- Starter - ENGAGE

- Throttle:  800 – 1200 RPM

- Oil pressure – CHECK

- Avionics Master Switch – ON
Radios – SET & CHECK
Transponder – SQUAWK STBY

- Electrical Equipment – ON

- Fuel Pump OFF

STARTING ENGINE FLOODED

- Brakes – SET

- Fuel selector – DESIRED TANK

- Throttle –  OPEN ½  INCH

- Mixture – FULL RICH

- Carburetor heat – FULL COLD

- Master Switch – ON

- Electric fuel pump – ON

- Propeller Area – CLEAR

- Starter - ENGAGE

- Throttle: 800 – 1200 RPM

- Oil pressure – CHECK

- Avionics Master Switch – ON
Radios – SET & CHECK
Transponder – SQUAWK STBY

- Electrical Equipment – ON

- Fuel Pump OFF

EXTERNAL POWER SOURCE START

- Master Switch – OFF

- All Electrical Equipment – OFF

- Terminals – Connect

- External Power Plug – insert in fuselage

- Proceed with normal start, MASTER SWITCH OFF

- Throttle – lowest possible RPM

- External power plug – disconnect from fuselage

- Master Switch – ON Check ammeter

SHORT FIELD TAKEOFF

- Flaps – 25 degrees (second notch)

- Carburetor Heat – COLD

- Throttle – FULL OPEN

- AT 41-49 KIAS – BACK PRESSURE ROTATE TO CLIMB ATTITUDE

- AFTER LIFT OFF – ACCELERATE TO 45-54 KIAS BASED ON WEIGHT

- ACCELERATE TO VX = 64 KIAS

- Flaps – SLOWLY RETRACT THE FLAPS PAST THE OBSTACLE.

- ACCLERATE TO VY=76 KIAS

SOFT FIELD TAKEOFF

- Flaps – 25 degrees (second notch)

- Carburetor Heat – COLD

- Throttle – FULL OPEN

- AT 41-49 KIAS – BACK PRESSURE ROTATE TO CLIMB ATTITUDE

- AFTER LIFT OFF – ACCELERATE TO 45-54 KIAS BASED ON WEIGHT

- ACCELERATE TO BEST FLAPS UP ANGLE  OF CLIMB SPEED – 64 KIAS

- ACCLERATE TO BEST FLAPS UP RATE OF CLIMB SPEED – 76 KIAS

- Flaps – SLOWLY RETRACT THE FLAPS PAST THE OBSTACLE.

- ENROUTE CLIMB

- Airspeed – 87 KIAS

- Throttle – FULL OPEN

- Mixture – RICH (above 5000 feet lean to obtain maximum RPM)

DESCENT – POWER OFF

- Carburetor Heat – ON IF REQD

- Throttle – CLOSED (IDLE)

- Airspeed – AS REQUIRED

- Mixture – LEANED AS REQUIRED

- Power – VERIFY WITH THROTTLE EVERY 30 SECONDS

SOFT FIELD LANDING

- Fuel Selector – FULLEST TANK

- Seat Backs / Belts / Harness

- Electric Fuel Pump – ON

- Mixture –  FULL RICH

- Carburetor Heat – ONLY IN SUSPECTED ICING CONDITIONS

- Flaps – SET – (VFE= 102 KIAS)

- Trim TO 75 KIAS

- Final approach: flaps 40°@66 KIAS

- Power – MAINTAIN UNTIL AFTER TOUCHDOWN

- After touchdown – NOSE UP AS LONG AS POSSIBLE

- After nose down – NO BRAKES – KEEP PLANE MOVING

SHORT FIELD LANDING

- Fuel Selector – FULLEST TANK

- Seat Backs / Belts / Harness

- Electric Fuel Pump – ON

- Mixture –  FULL RICH

- Carburetor Heat – ONLY IN SUSPECTED ICING CONDITIONS

- Flaps – SET – (VFE= 102 KIAS)

- Trim TO 75 KIAS

- Final approach: flaps 40°@66 KIAS

- After touchdown – NOSE UP, FLAPS UP,

- After nose down – APPLY BRAKES WITHOUT SKIDDING

| Airspeed IND | V (KIAS) |  |
| --- | --- | --- |
| Stall (Landing) | VSO | 49 |
| Stall (Clean) | VS | 55 |
| Best Angle | VX | 64 |
| Best Rate | VY | 76 |
| Flaps | VFE | 102 |
| Maneuver | VA | 113 |
| Normal | VNO | 125 |
| Never Exceed | VNE | 154 |
| Best Glide |  | 76 |
| Initial Approach Speed |  | 76 |
| Final Approach Speed |  | 66 |

If engine does not start within 10 seconds, prime and repeat